LESSON 4 READING MEASUREMENTS

Objective:

At the end of this lesson, the students should be able to:

Read and interpret measurements accurately using common measuring tools and identify whole-number, fractional, decimal, metric, and English measurements.

## READING COMMON MEASURING TOOLS

### Introduction

In carpentry, accurate measurement is essential before cutting, marking, or assembling materials. A carpenter must be able to read measuring tools correctly to ensure that materials are prepared according to the required size.

Common measuring tools used in carpentry include the tape measure, steel rule, try square, and vernier caliper.

- Reading Common Measuring Tool

## A.  PULL PUSH RULE

A tape measure is a flexible measuring tool used to determine the length, width, height, and distance of materials.

It commonly contains both metric and English measurements.

### Parts of a Tape Measure

1. Blade – the long metal strip containing the measurement markings.

2. Hook – the metal piece at the end of the blade used to catch the edge of an object.

3. Case – the housing that stores and protects the blade.

4. Lock – the mechanism used to hold the blade in position.

### How to Read a Pull Push Rule

### Step 1: Place the hook at the starting edge of the material.

Step 2: Extend the blade along the material.

Step 3: Keep the blade straight and properly aligned.

Step 4: Identify the number closest to the end of the material.

Step 5: Count the smaller markings after the whole number.

Step 6: Combine the whole number with the appropriate fraction or decimal.

Step 7: Record the measurement together with its unit.

### Example

If the end of the material is halfway between 5 inches and 6 inches:

Measurement = 5 ½ inches

# B. STEEL RULE

A steel rule is a rigid metal measuring tool used to measure and mark short lengths accurately.

It may have metric measurements, English measurements, or both.

### Parts of a Steel Rule

1. Graduations – the lines and numbers that indicate the measurement.

2. Edge – the straight side used as a guide when measuring or marking.

3. End – the end of the rule used as the starting point of the measurement.

### How to Read a Steel Rule

Step 1: Place the end of the rule at the starting point.

Step 2: Make sure the rule is straight and properly aligned.

Step 3: Look at the measurement markings.

Step 4: Identify the marking where the material ends.

Step 5: Read the measurement.

Step 6: Record the measurement with the correct unit.

Example

If the edge of the material reaches the 15 cm mark:

Measurement = 15 cm

If it reaches the 18 cm and 5 mm mark:

Measurement = 18.5 cm

# C. TRY SQUARE

A try square is a tool used mainly for checking and marking 90-degree angles on wood and other materials.

It helps ensure that edges and lines are straight and square.

### Parts of a Try Square

1. Stock – the thicker part placed firmly against the edge of the material.

2. Blade – the thin, straight metal part used as a guide for marking and checking.

### How to Use a Try Square

Step 1: Place the stock firmly against the edge of the material.

Step 2: Make sure the stock is flat against the surface.

Step 3: Hold the try square securely.

Step 4: Use the blade as a guide when making a mark.

Step 5: Check that the marked line forms a 90-degree angle with the edge.

### Example

If you need to mark a line across a piece of wood, place the stock against the edge and mark along the blade.

The line should form a 90° angle with the edge.

Remember: A try square is mainly used for checking squareness and marking, rather than measuring length.

## D. Vernier Caliper

A vernier caliper is a precision measuring tool used to measure small objects accurately.

It can be used to measure outside dimensions, inside dimensions, and depth.

### Parts of a Vernier Caliper

1. Main Scale – the fixed scale containing the main measurement markings.

2. Vernier Scale – the movable scale used to obtain a more precise reading.

3. Outside Jaws – used to measure the outside diameter or thickness of an object.

4. Inside Jaws – used to measure the inside diameter of an opening.

5. Depth Rod – used to measure the depth of holes or openings.

### How to Read a Vernier Caliper

### Step 1: Place the object between the appropriate jaws.

### Step 2: Adjust the jaws until they make proper contact with the object.

### Step 3: Read the measurement on the main scale.

### Step 4: Read the additional measurement indicated by the vernier scale.

### Step 5: Combine the main-scale and vernier-scale readings.

### Step 6: Record the final measurement with the correct unit.

### Example

If the main scale reads 20 mm and the vernier scale adds 0.5 mm:

Measurement = 20.5 mm

# II. Types of Measurements

## A. WHOLE-NUMBER MEASUREMENTS

A whole-number measurement is a measurement expressed using a complete number without a fraction or decimal.

### Examples:

- 5 cm

- 10 cm

- 25 cm

- 30 inches

### Example

If the material reaches exactly the 20 cm mark:

Measurement = 20 cm

B. FRACTIONAL MEASUREMENTS

A fraction represents a part of a whole. Fractions are commonly found when reading measurements in inches.

### Common Fractions

- 1/2 inch

- 1/4 inch

- 1/8 inch

- 3/4 inch

- 3/8 inch

### Example

If the measurement is between 8 inches and 9 inches, and the mark is halfway between them:

Measurement = 8 1/2 inches

This means 8 whole inches and 1/2 inch.

## C. DECIMAL MEAUREMENTS

A decimal measurement uses a decimal point to express a part of a unit.

### Examples:

- 5.5 cm

- 10.25 cm

- 15.5 mm

- 20.75 inches

### Example

If the measurement reads 12.5 cm:

Measurement = 12.5 cm

Since:

0.5 cm = 5 mm

then:

12.5 cm = 12 cm and 5 mm

# III.READING METRIC MEASUREMENTS

# The metric system is commonly used for measuring materials.

### Common Metric Units

# Millimeter (mm) – used for small measurements.

# Centimeter (cm) – used for short measurements.

# Meter (m) – used for longer measurements.

### Important Conversions

# 10 mm = 1 cm

# 100 cm = 1 m

# 1,000 mm = 1 m

### Example

If a measurement is 25 cm, convert it to millimeters:

25 × 10 = 250 mm

Therefore:

25 cm = 250 mm

# IV. READING ENGLISH MEASUREMENTS

The English measurement system commonly uses inches and feet.

### Common Units

Inch (in.) – used for smaller measurements.

Foot (ft.) – used for longer measurements.

### Important Conversion

12 inches = 1 foot

### Example

If a piece of wood measures 3 feet, convert it to inches:

3 × 12 = 36 inches

Therefore:

3 feet = 36 inches

# V. Accuracy in Reading Measurements

Accuracy means getting the correct measurement without unnecessary mistakes.

When reading measurements:

• Start from the zero mark.

• Keep the measuring tool properly aligned.

• Read the markings carefully.

• Identify whether the measurement is metric or English.

• Check the smaller divisions.

• Write the correct unit.

### Example

Do not write:

25

Instead, write:

25 cm

because the unit tells us what the measurement represents.

# VI. Key Points to Remember

Pull Push Rule – used to measure length, width, height, and distance.

Steel Rule – used to measure and mark short lengths accurately.

Try Square – used to check and mark 90° angles.

Vernier Caliper – used to measure small dimensions precisely.

Whole Number – a complete number such as 20 cm.

Fraction – a part of a whole such as 5 1/2 inches.

Decimal – a measurement containing a decimal point such as 12.5 cm.

Metric – uses millimeters, centimeters, and meters.

English – uses inches and feet.

### Important Safety and Care Reminder

Always use the correct measuring tool for the job. Handle measuring tools carefully, keep them clean, and return them to their proper storage after use. Accurate measurement helps prevent errors in cutting, fitting, and assembling materials.